Content

1	Overall impressions from the analysis of the SEA for the Ekostoky programme	1

2	Major issues regarding the content and structure of the SEA-Report	1

2.1	Have the environmental and health authorities been consulted in the scoping process?	1

2.2	SEA-Report	2

# Overall impressions from the analysis of the SEA for the Ekostoky programme

We consider the SEA for the Ekostoky programme as an ambitious effort to implement the requirements resulting from the Ukraine law and guidelines. In the formal dimension, the report reflects both regulatory documents thoroughly. It also partly makes use of appropriate methods for the assessment of the likely different environmental effects of the measures of the programme and addresses possible mitigation measures and alternatives.

Nevertheless, we identified several weaknesses, mostly in the technical and methodological dimension. These are rooted in the lack of a clear and systematic methodology on how the SEA can be conducted appropriately, which is also not provided be the Ukraine law and guidelines. One of the difficulties may also be caused by the character of the Ekostoky programme as a programme which targets basically at the improvement of environmental and health objectives (other than for example power plants or industrial facilities where it is clear that an environmental assessment is required).

Our preliminary conclusion is, that there is a need to improve the regulatory framework for the Ukraine in order to better align it to the EU requirements and to provide a more systematic and elaborated methodology for a more effective application.

# Major issues regarding the content and structure of the SEA-Report

In the following, we present key findings from the detailed analysis. For more details, please see the file 2_Synthesis table Legislative Review and Ekostoky Assesment.xlsx

## Have the environmental and health authorities been consulted in the scoping process?

  - Issue: Authorities shall be consulted (EU directive) and shall submit their comments and proposals in writing with a statement on determining the date of application receipt (LAW 2354) No comments or consultations are mentioned in the SEA or scoping document.

  - Recommendation: Authorities (Environmental and Public health at least) should be actively consulted in line with the EU directive and handbook. It could be stated, if they do not want to comment on the scoping document or to be involved in the SEA process.

## SEA-Report

In the following, we present key findings from the detailed analysis. For more details, please see the file Analytical-grid_V6.xlsx

- Have the environmental and health authorities been consulted in the scoping process?

  - Issue: Authorities shall be consulted (EU directive) and shall submit their comments and proposals in writing with a statement on determining the date of application receipt (LAW 2354) No comments or consultations are mentioned in the SEA or scoping document.

  - Recommendation: Authorities (Environmental and Public health at least) should be actively consulted in line with the EU directive and handbook. It could be stated, if they do not want to comment on the scoping document or to be involved in the SEA process.

- SEA-Report

  - Chapter 2 to 5:

    - The structure and content of these chapters are in line with the requirements of law2354 and the guideline 705 except minor adjustments

    - Issue: But the overall line of argumentation is not from environmental objectives (national/international) to environmental issues (past and future trends) and to systematically linked appropriate environmental indicators, see the recommendations in the grid V6.

    - Recommendations:

      - Some content esp. environmental objectives (national and international) should be the starting point and then integrated into the discussion of the state of the environment, past and future trends and on which environmental indicators (quantitative or qualitative) the analysis should focus.

      - The first impact assessment of the program measures in Chapter 4, which has many really good and differentiated questions (table 4.1) should be integrated into the assessment of likely significant impacts of the program measures in chapter 6

  - Chapter 6:

    - Issues:

      - The Chapter is in line with the Law 2354 and the Guidelines 705, but the issues here are the same as stated with those two documents.

      - First only a very broad and general matrix is presented but not even discussed which types of impacts are likely on four different area types (production, settlement, communal and landscape/recreational). Some symbols are not in the legend, but that can be due to translation.

      - The impact of each measure on each environmental component is assessed by describing it qualitatively, which is good und how it should be done. The impact qualification (+2,+1/0/-1,-2 from the guidelines 705) is missing. And also the link to proper environmental indicators is missing.

      - The cumulative Impact is again rather broad and a repetition of the program goals and expected results.

      - Overall, no interdependencies/causal relationships/assumptions are established, how the measures impact all environmental issues and not only those environmental issues/problems like wastewater treatment

    - Recommendations:

      - Regarding the EU handbook for EU plans and programs...what should be done here

        - For each measure in the program -the effect on the environment and on env. indicators should be established

        - -alternatives and measures to prevent, reduce etc. effects should be discussed

        - Afterwards cumulative effects should be assessed which can change individual measure assessments

        - This should be the basis for final program changes

        - The Evaluation of selection criteria for activities or projects should be integrated, too (like partly down in table 7.1)

        - The really good questions from Table 4.1 from Chapter 4 should be integrated here, when discussing the measures!

        - Alternatives, Measures to mitigate and cumulative effects could be discussed separately, but that makes it much harder to link them directly to the effects of the program measures, which is the method to prefer

        - A general assessment of the impact of all measures on each environmental issues is not necessary, but could be an useful addition.

  - Chapter 7 and 8:

    - Issues: Measures to mitigate:

The measures are mostly those of the programm measures itself. Only 5 or 6 those measures (of 25) are envisaged in addition to mitigated negative implementation effects...

    - Issues: Alternatives:

the alternatives are taken from the program itself and are not new), but without discussing the possible environmental impacts of the alternative, but rather the optimal one with the most positive effects.

    - Recommendations: Mitigation Measures

In our view the 5 to 6 additional measures to mitigate negative impacts of the program measures should be discussed here (linked to the implementation), not measures of the program itself that have likely positive impacts on the environment. The focus should be on finding more of those additional mitigation measures...

    - Recommendations: Alternatives

In our view this is a wrong understanding of alternatives. The alternatives presented in the program document which have be incorporated into the SEA are reasons and justifications why the program should be implemented. In the SEA-Report alternatives to the proposed measures, which have an even more positive (or less negative) impact on the environment shall be discussed! That is also in line with the 705 guidelines.