| Item No. | Item Description (Refuse Collection vehicle for collection of municipal solid waste, 8 m3 capacity) 4 Units The vehicle must comply with Machinery Directive 2006/42EG or equivalent and be approved by an accredited EC type-examination body |
| --- | --- |
| 1 | General requirements |
| 1.1 | New vehicle: The Vehicle to be procured must be new: This is assumed if – no more than 12 month have been elapsed between the production of the Vehicle (Chassic and the body structure) and the transfer of ownership of the Vehicle to GIZ The vehicles have not been used in road traffic and off-road (outside the production site) and do not exceed a mileage of 1500 km The manufacturer's specified standard equipment for the vehicle model offered is part of the specification and must therefore be included in the delivery |
| 1.2 | Number of axles: 2 |
| 1.3 | Gros Vehicle Weight 14 tonnes or above |
| 1.4 | It is required that at minimum 4.8 – 6 tonnes tonnes of waste can be collected. |
| 2 | Engine |
| 2.1 | Fuel type: Diesel engine (Euro 5) |
| 2.2 | At least 4 cylinders, direct injection |
| 2.3 | Engine capacity: 2800 cc- 8000 cc |
| 2.4 | Engine power (minimum): Not less than 180 hp |
| 2.5 | Fuel tank volume (minimum): 90 liters |
| 2.6 | Turbocharged, |
| 3 | Transmission |
| 3.1 | Automated or Automatic gearbox without clutch pedal. |
| 3.2 | PTO (Power Take off and RPM (Revolution per Minute) limit (<1200) |
| 4 | Cabin |
| 4.1 | Number of seats: 1 driver seat + 1 or 2 passenger seats 2 + 1 preferred |
| 4.2 | Central Door looking system |
| 4.3 | Two keys (Min) |
| 4.4 | Air conditioning (cooling and heating) Required CFC free |
| 4.5 | Cabin specs (Driver plus two crew seating) Audio basic |
| 5. | Braking system |
| 5.1 | Front disc brakes, rear disc or drum brakes (for both) |
| 5.2 | ABS brake system or ESP (Electronic Stability Program) |
| 5.3 | Air Brake system |
| 5.4 | Engine Brake or Exhaust Brake or equivalent |
| 6 | Suspension |
| 6.1 | Front suspension - coil springs, hydraulic shock absorbers |
| 6.2 | Rear suspension - coil springs, hydraulic shock absorbers |
| 7 | Steering system |
| 7.1 | Front wheels - ordinary, rear wheels – dual |
| 7.2 | Tires of same size per axle (front axle same size, rear axle same size) |
| 7.3 | Power Steering |
| 8 | Compaction equipment, hydraulic system (To be changed completely) |
| 8.1 | Waste collection unit in rotary drum design with hydraulic drive. |
| 8.2 | Compactor useful volume (approx.): 8-10 m3 |
| 8.3 | compaction ratio minimum 5:1 |
| 8.4 | Cylindrical collection container made of high-quality steel sheet E335 or similar |
| 8.5 | Strength 570-710 N/mm² |
| 8.6 | Thickness min. 3mm |
| 8.7 | Diameter 2200mm |
| 8.8 | Bearing of the drum container at the rear with one integral radial-axial bearing that covers the entire outer diameter of the drum, and at the front via a centrally located hub and one radial-axial bearing supported on the frame |
| 8.9 | Two- flight screw conveyors for waste transport in the collection container |
| 8.10 | Rotation drive: hydrostatic closed circuit - hydraulic pump with a maximum loading pressure of at least 380 bar driven via a cardan shaft and an electro-magnetic coupling |
| 8.11 | Press screw covered with highly wear-resistant, bolted plates. Quickly and individually replaceable without disassembling the rear section. |
| 8.12 | Tailgate phosphated, primed and coated with a single color 2K paint |
| 8.13 | Hydraulic system for rear loading of Euro type containers - suitable for 120,240,770 and 1,100 liters. |
| 8.14 | Lifting system details: two-stage |
| 8.16 | The lifter works on the principle of translatory lifting and rotary emptying |
| 8.17 | Loading hopper capacity: 0,8-1 m3 |
| 8.18 | Loading and unloading of waste from the drum container only by changing the direction of rotation |
| 8.19 | Rear door operations: lifting, lowering, locking and unlocking – with the help of hydraulic cylinders |
| 8.20 | Sewage water tank under tailgate with outlet |
| 8.21 | Hydraulic safety sensors |
| 8.22 | Low hydraulic oil level safety sensor – compaction unit stops automcatically |
| 9 | Electrical system |
| 9.1 | Electrical system 12–24V, 2 beacon lamps |
| 9.2 | Orange beacon |
| 9.3 | Emergency stop systems /Control box and emergency stop at tailgate |
| 9.4 | Side marker lights |
|  | Possibility of managing the superstructure operations from the cabin using a multifunctional control panel with operation indicators, as well as control of operations from the side of the vehicle with the option of two-handed lowering for safety |
| 9.5 | The rear camera is connected to the same control panel and transmits the image on a minimum screen size of 7” |
| 9.6 |  |
| 10 | Additional equipment |
| 10.1 | First aid kit |
| 10.2 | Extinguisher, minimum 8 kg |
| 10.3 | Reflective triangle |
| 10.4 | Warning lamp |
| 10.5 | Hydraulic jack |
| 10.6 | Tire inflation pump and hose |
| 10.7 | Tool set |
| 10.8 | 2 safety wedges |
| 10.9 | Spare wheel |
| 10.10 | Foldable anti-skid footboards |
| 10.11 | Self-lubrication system for compaction device. |
| 11 | Other requirements |
| 11.1 | Service book and warranties in Engl. language |
| 11.2 | Spare parts catalog with diagrams for hydraulic and electrical systems |
| 11.3 | Color requirement: White body RAL 9010 |
| 11.4 | Necessary spare parts list included |
| 12 | Warranty and /Sales Service /Delivery: |
| 12.1 | Accompanying documents: Certificate of origin, design copy including dimensions, materials of construction, standards, etc. |
|  | The model offered has been approved by the manufacturer for use in the country of destination Jordan |
| 12.2 | Repair and maintenance services, as well as spare parts, for the vehicle/model offered, are available in Jordan, the country of use. This includes (in particular): - Maintenance services of the delivered vehicle/model in accordance with the manufacturer's specifications. - Repair services of the delivered vehicle/model in accordance with the manufacturer's specifications - Provision of OEM spare parts for the delivered vehicle/model Bidders are required to expressly confirm compliance with this requirement in their bid (PDF) and provide the addresses of the service centers and distribution partner(s) in Jordan |