NDC-TEC – RFP for Turn-Key Electric Boats (2 units) and Electric Propulsion Systems (6 units)

Table of Contents

1	Abbreviations	2

2	Project Context and Background	4

3	E-boat General Tender Requirements	5

4	Lot A: Coast Guard Turnkey Vessel Specifications (2 units)	6

4.1.1	Hull & General Performance Requirements for Lot A	6

4.1.2	Outboard Motor Requirements for Lot A	8

4.1.3	Marine Battery Specifications for Lot A	10

4.1.4	Integrated Mission Equipment for Lot A	11

5	Lot B: Electric Propulsion Systems for 24-ft Vessels (3 Units)	12

5.1.1	General Requirements for Electric Propulsion Systems for Lot B	12

5.1.2	Outboard Motor Specifications for Lot B	12

5.1.3	Marine Battery Specifications for Lot B	14

5.1.4	Control System Specifications for Lot B	15

5.2	Propulsion Systems Installation and Commissioning Lot B	16

5.2.1	Physical Details for 24-ft vessels to be Outfit with Electric Propulsion in Lot B	16

5.2.2	Positioning and Mounting of Battery and other components Lot B	17

5.2.3	Considerations for Electrical and Communications Installation Lot B	17

5.2.4	Provision of Boat Hulls for Installation of Propulsion Systems (Lot B)	18

6	Lot C: Electric Propulsion Systems for 20-ft Vessels (3 Units)	18

6.1.1	General Requirements for Electric Propulsion Systems for Lot C	18

6.1.2	Electric Outboard Requirements for Lot C	18

6.1.3	Marine Battery Specifications for Lot C	20

6.1.4	Control System Specifications Lot C	21

6.2	Propulsion Systems Installation and Commissioning Lot C	22

6.2.1	Physical Details of 20-ft Vessels to be Outfit with Electric Propulsion Lot C	22

6.2.2	Positioning and Mounting of Battery and other components Lot C	22

6.2.3	Considerations for Electrical and Communications Installation Lot C	23

6.2.4	Provision of Boat Hulls for Installation of Propulsion Systems (Lot C)	23

7	Training (All Lots)	23

7.1	Lot A – Training Requirements	23

7.1.1	Driver Training (Practical)	24

7.1.2	Operator Training (Operations and Depot)	24

7.1.3	Services at the place of assignment:	24

7.2	Lot B and C – Training requirements	25

8	Shipping and Logistics (All Lots)	25

9	Project Delivery Times	26

9.1.1	Delivery Time - Lot A	26

9.1.2	Delivery Time - Lot B and Lot C	26

10	After-sales support (All Lots)	26

11	Acceptance	27

List of Tables

Table 1- Summary of Tender Lots	4

Table 2 – Hull and General Performance Requirements for Lot A – Turnkey Patrol Vessel for Grenada Coast Guard	5

Table 3 – Outboard Motor Requirements for Lot A – Turnkey Electric Vessels for Grenada Coast Guard	6

Table 4 – Marine Battery Specifications for Lot A – Turnkey Vessels	7

Table 5 – Integrated Mission Equipment for Lot A – Turnkey Vessel for Grenada Coast Guard	8

Table 6 - General Requirements for Propulsion Systems for Lot B	9

Table 7 – Electric Outboard Motor Requirements – 24ft vessel	9

Table 8 – Marine Battery Specifications – Vessel Propulsion Systems	10

Table 9 – Control System Specifications	11

Table 10 - General Requirements for Propulsion Systems for Lot C	14

Table 11 – Electric Outboard Motor Requirements for Lot C – 20ft vessel	14

Table 12 – Marine Battery Specifications for Lot C – Vessel Propulsion Systems	15

Table 12 – Control System Specifications	16

List of Figures

Figure 1 - 24 Foot Vessel General Arrangement	12

Figure 2 - 20 Foot Vessel General Arrangement	17

# Abbreviations

ABYC 		American Boat and Yacht Council

CAD 		Computer Aided Drawings

CCREEE	Caribbean Centre for Renewable Energy and Energy Efficiency

CE		Conformité Européenne – European Certification

GIZ		Gesellschaft für internationale Zusammenarbeit (the German governmental 		agency for international cooperation)

IMO		International Maritime Organization

kW 		kilowatt

kWh 		kilowatt-hour

Nm 		Newton-meters

NM		Nautical Miles

NMMA		National Marine Manufacturer’s Association

SoC 		State of Charge

USCG 		United States Coast Guard

# Project Context and Background

The project “Supporting the implementation of NDCs in the Caribbean - transforming the transport and energy sectors towards a low-carbon and climate-resilient future” (NDC-TEC), funded by the German Federal Ministry for the Environment, Climate Action, Nature Conservation and Nuclear Safety through the International Climate Initiative (IKI) aims to support CARICOM Member States in enhancing the ambition of their NDCs and in implementing transformative actions within the energy and transport nexus. It offers tailored support to Antigua & Barbuda, Belize, Grenada, Jamaica, and Saint Lucia in accelerating the implementation and raising the ambition of their NDCs in the energy and transport sectors. Strategic access to climate financing for NDC implementation is facilitated. In Grenada, Jamaica and Saint Lucia (“flagship countries”), flagship projects introduce new and market-disruptive technologies, which lead to long-term climate change mitigation and increased resilience. The flagship projects comprise technology demonstrations, capacity building and enabling frameworks for a long-term sectoral shift and serve as a model for further uptake in the region. Through knowledge products, dissemination and regional outreach, all CARICOM member states will benefit from good practices and are encouraged to position themselves as leaders for comprehensive and innovative NDC implementation. The project is implemented by Climate Analytics, CCREEE, CDB, UWI and GIZ as consortium lead.

Many CARICOM state remains heavily dependent on imported fossil fuels for transport and marine activities. Therefore, the Grenada flagship, implemented by GIZ and CCREEE, seeks to demonstrate the feasibility of electric propulsion in the marine sector as a pathway to reducing greenhouse gas emissions, lowering operating costs, and strengthening climate resilience. The Grenada flagship focuses on introducing electric propulsion for boats, complemented with shore-based solar charging infrastructure.

# E-boat General Tender Requirements

GIZ seeks to procure three distinct lots of hardware to be deployed in Grenada, Carriacou, and Petit Martinique. Proponents have the option to submit offers for a single lot or as much lots as they can potentially offer. Each lot will be evaluated separately. As such a proponent can be awarded 1 lot but not the other. Lots are summarized in table 1 below:

Table 1- Summary of Tender Lots

| Lots | Details | Quantity |
| --- | --- | --- |
| A | Complete Turnkey Electric Patrol Vessels Complete, ready-to-operate electric boats for the Royal Grenada Coast Guard. - 84 kWh Battery - ≥ 85 KW Continuous Power Electric Motor - Integrated Coast Guard Mission Equipment - Commissioning and Training | 2 complete vessels |
| B | Electric Propulsion Systems for 24 ft Fishing Vessels Electric outboards, battery and control system - 110 kWh Battery - ≥ 85 KW Continuous Power Electric Motor - Control System - Installation & Training | 3 propulsion systems |
| C | Electric Propulsion Systems for 20 ft Fishing Vessels Electric outboards, battery and control system - 110 kWh Battery - = 60 KW Continuous Power Electric Motor - Control System - Installation & Training | 3 propulsion systems |

The supplier is requested to fill out the specification tables of the following pages under the columns “Requirement Met” and if necessary under “Supplier comments on Requirements”.

# Lot A: Coast Guard Turnkey Vessel Specifications (2 units)

GIZ seeks to procure two (2) complete electric vessels (with an option for a 3rd vessel), which will be used for patrolling the coastal areas around Grenada. The typical use will involve trailering to the location (on land) for that day, launching and then patrolling at trolling speeds for 4-8 hours. Operations will be in all conditions in which the boat can safely operate and all year round.

The vessels shall be turn-key, delivered to Grenada, completely built and tested according to the certification requirements of the standard to which the vessel is certified (ABYC, NMMA, CE or similar from an IMO recognised organization). A safety, build and electrical wiring standards certificate from the relevant standards organization (i.e. ABYC, NMMA, CE, IMO or any IMO recognized organization) shall be supplied with the vessel to confirm. The propulsion systems onboard the vessels shall meet the general requirements as outlined in this section.

### Hull & General Performance Requirements for Lot A

Table 2 – Hull and General Performance Requirements for Lot A – Turnkey Patrol Vessel for Grenada Coast Guard

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Operation Role | Suitable for low-speed (4-8 knots) coastal patrol, search and rescue (SAR), and marine law enforcement interventions. |  |  |
| Hull Type | Deep-V Rigid Inflatable Boat (RIB), "Ribbed Hull", or fully molded Deep-V Fiberglass Hull. |  |  |
| Hull Material | Fiberglass (FRP): Marine-grade, glass-reinforced plastic utilizing premium vinyl ester resins for osmosis protection and heavy-duty structural stringer grids. Aluminium Alternative: High-tensile, marine-grade 5052 or 5086 powder-coated aluminium (minimum 4mm thickness on hull bottom). Unreinforced hulls are strictly unacceptable. |  |  |
| Tube Material (If RIB Option) | Heavy-duty commercial-grade Hypalon (minimum 1670 Dtex) or marine grade polyurethane. PVC tubes are not acceptable. Inflatable collars must feature a minimum of five (5) independent, airtight chambers with flush-mount pressure relief valves. |  |  |
| Dimensions | Length Overall (LOA): 20ft to 24ft (6.0m to 7.3m). Beam: 7.5ft to 9ft (2.28 to 2.75m) |  |  |
| Deadrise | 20° to 25° (Deep-V hull profile) at the transom to ensure high-speed tracking and impact mitigation in rough coastal chop. |  |  |
| Crew Capacity & Payload | Minimum Passenger Seating: Dedicated seating for six (6) personnel. Minimum Payload Capacity: Must support a minimum operational payload of 1,800 lbs (816 kg), encompassing crew, gear, and tactical equipment. |  |  |
| Towing Post | Aft low-post heavy-duty towing bit or reinforced samson post, structurally tied directly into the hull grid framework (316 Stainless Steel or marine-grade aluminium) rated for towing vessels of equivalent displacement (up to 5000 lbs) |  |  |
| Seating & Ergonomics | Seats shall be minimum 28 oz.UV-stabilised (min 1000 hours), pink-stain/antifungal resistant marine vinyl over high density foam core, assembled with Type 316 stainless steel fasteners, and structural brackets to ensure saltwater corrosion resistance. |  |  |
| Canopy / T-Top | Must come equipped with a frame-mounted T-Top or canopy providing permanent sun and weather protection for the helm and passengers. Structure must be marine-grade anodized aluminium or 316 stainless steel, engineered to withstand speeds up to 45 knots without structural deflection. |  |  |
| Self-Righting & Stability Capabilities | The vessel is not required to be fully self-righting, but must exhibit high static and dynamic stability. Powertrain battery banks must be housed beneath the deck center-line to achieve an exceptionally low center of gravity (CoG). The vessel must meet or exceed ISO 6185 (Category B) or equivalent international stability and buoyancy standards under full load. |  |  |
| Safety & Buoyancy Standards | Inherent Buoyancy: The hull structure must incorporate positive flotation (e.g., closed-cell injected foam core within the fiberglass stringer grid or multi-chambered RIB collars) ensuring the vessel remains afloat, level, and stable even when completely swamped or with two adjacent chambers compromised. |  |  |
| Floor / Deck Material | Material Class: Premium, UV-resistant, non-absorbent Ethylene-Vinyl Acetate (EVA) foam decking or a dual-density closed-cell PE/EVA foam blend securely bonded to the deck. Texture & Traction: Surface must feature a brushed faux-teak or diamond-grooved non-slip texture maintaining high traction when completely wet or exposed to oils. |  |  |
| Deck Drainage Features | Gravity Deck-Shedding: The cockpit floor sole must be entirely elevated above the static waterline at maximum displacement to ensure a fully self-draining deck via gravity. Evacuation Hardware: The transom must incorporate dual, high-capacity (1.5 inch or larger), flush-mount non-return valves (scuppers) or elephant-trunk style drains capable of rapidly shedding green water without allowing backflow. |  |  |
| Bilge & Sump Features | Dual-Zone Drainage: Sub-floor bilge compartment must be equipped with an automatic submersible bilge pump rated for a minimum of 750 GPH (Gallons Per Hour), driven by an electronic water sensor. Inspection & Redundancy: Deck layout must include a dedicated, gasketed, watertight bilge access hatch for manual inspection, alongside a secondary manual diaphragm hand pump accessible from the helm position. |  |  |
| Certifications & Build Standards | The vessel construction, electrical wiring, and safety systems must comply with and be certified to one or more of the following recognized standards: CE Category B (Offshore) / ISO equivalents, ABYC standards, NMMA certification, or Transport Canada Construction Standards (TP 1332) or referenced equivalents from these standards. Bidders must supply structural drawing approvals stamped by a registered Professional Engineer (P.Eng. / P.E.) or supply a survey validation performed and signed off by an accredited marine surveyor holding a valid SAMS (AMS) or NAMS (CMS) designation, or an authorized CE Notified Body." |  |  |
| Warranty | Minimum of a five (5) year warranty/ manufacturer guarantee on the hull/deck framework and a minimum of a three (3) year warranty/manufacturer guarantee on inflatable tube materials/seams under commercial/government operational use. The 2-year warranty according to “General Purchase Conditions of GIZ” remains unaffected of the above-mentioned warranty requirements. |  |  |

### Outboard Motor Requirements for Lot A

Table 3 – Outboard Motor Requirements for Lot A – Turnkey Electric Vessels for Grenada Coast Guard

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Power | Minimum acceptable: ≥ 85 kW Continuous Power rating. |  |  |
| Torque | Minimum acceptable: ≥ 275 Nm Include a torque curve that includes continuous and peak torque capabilities. Continuous means 60 minutes of operation. |  |  |
| Dimension limitations | Dimensions must be comparable to those of a standard gasoline outboard engine – no greater than double the size |  |  |
| Motor Efficiency | Motor should be sized appropriately to comfortably achieve operational range based on the proposed battery complying with requirements of section 4.1.3 and general vessel operational parameters. Provide efficiency map and QA tests data for the outboard motor so that this can be validated by the technical evaluation committee. |  |  |
| Material | System must be designed for ocean/saltwater use. IP 67 ingress rating or better. |  |  |
| Mounting | Standard outboard mounting required per ABYC TH-12 Standard or ISO 12215-9 series standards. |  |  |
| Propellor | Propeller match system and must perform ≥5000 before replacement required |  |  |
| Safety or Performance Standards | System must meet current ABYC standards including E-30 and A-31 or equivalent standard. All standards must be listed. Only internationally recognised standards issued by an IMO Recognized Organisation will be accepted |  |  |
| Motor Warranty | Warranty period measured from acceptance of vessel (signed “Certificate of Acceptance”). Minimum of a 2-year warranty according to “General Purchase Conditions of GIZ” plus an additional manufacturer guarantee of minimum 1-year must be provided on all core manufacturer-supplied components of the outboard unit. Including but not limited to: - The Electric Motor & Power Electronics: Stator, rotor, internal bearings, and the integrated motor controller/inverter unit housed within the outboard enclosure. - The Drivetrain & Lower Unit: The main drive shaft, internal gearcase components, propeller shaft, and all factory-installed internal water/oil seals. - Integrated Auxiliary Systems: Factory-installed internal cooling systems (including integrated pumps and heat exchangers) and the power trim/tilt hydraulic assembly and actuator." |  |  |

### Marine Battery Specifications for Lot A

Table 4 – Marine Battery Specifications for Lot A – Turnkey Vessels

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Battery Capacity | ≥ 84 kWh for each of the two (2) complete turn-key vessels. Please state size of the battery packs proposed. |  |  |
| Expandable Battery Pack | Battery pack should be capable of expansion by addition of additional packs in parallel to increase capacity |  |  |
| Enclosure Protection Rating | IP67 or better enclosure |  |  |
| Fire Rating | A0 or better fire rating |  |  |
| Marine Standards | ABYC E-30 and A-31 or equivalent standard. Only internationally recognised standards issued by an IMO Recognized Organisation will be accepted. |  |  |
| Battery Thermal and Safety Management System | Battery should have active thermal and safety management system to maintain battery temperature in optimal temperature range appropriate for operation in Tropical Caribbean Climate. An overview of the thermal management system shall be provided, with a reference to the standards applied. The thermal management system must meet the requirements of an Internationally recognized standard issued by IMO Recognized Organization. |  |  |
| Sensors | Thermal monitoring sensors, smoke sensors and fire sensors are required. |  |  |
| Fire Control and Management Procedure | Fire management and emergency procedures must be described. |  |  |
| Battery Warranty | Warranty period measured from acceptance of vessel (signed “Certificate of Acceptance”). Minimum of a 2-year warranty according to “General Purchase Conditions of GIZ” plus an additional manufacturer guarantee of minimum 1-year must be provided on all core manufacturer-supplied components of the outboard unit. Including but not limited to: - The Battery Management System (BMS): All internal and external BMS hardware, balancing circuitry, temperature/voltage sensors, and proprietary control software/firmware. - Cells and Structural Modules: All individual lithium cells, internal busbars, module interconnects, and the sealed, waterproof (IP-rated) battery enclosure. - Safety and Switching Components: Internal high-voltage contactors, fuses, manual disconnect switches, and integrated thermal management components (e.g., internal cooling plates or heat exchangers). |  |  |

### Integrated Mission Equipment for Lot A

The vessels must be delivered fully equipped with the following "Need" items as identified by the Coastal Guard:

Table 5 – Integrated Mission Equipment for Lot A – Turnkey Vessel for Grenada Coast Guard

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Navigation/Comms | VHF Radio, GPS, Depth Sonar |  |  |
| Safety/Rescue | Life vests, Flares, Fire extinguisher, Rope for Man Overboard, Foot pump |  |  |
| Signalling | Blue Police Lights, Strobe Light, Loudspeaker, Running Lights, Anchor Light |  |  |
| Deck Gear | Anchor |  |  |

# Lot B: Electric Propulsion Systems for 24-ft Vessels (3 Units)

GIZ seeks to procure all components required for three electric propulsion systems to be installed on newly built vessels used for fishing and transporting people. Three (3) systems will be used on 24ft vessels. The vessels will be used primarily for fishing. Fishing vessels require low-speed trolling for 6-8 hours at under 6 knots speed, with short distance, high-speed transit legs at up to 15 Knots speed (typically less than 5 nautical miles from shore). The vessels will be newly constructed, and the following systems will be needed for installation on the vessels:

### General Requirements for Electric Propulsion Systems for Lot B

Table 6 - General Requirements for Propulsion Systems for Lot B

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Propulsion | One (1) Electric Outboard Motor As described in sections below |  |  |
| Energy | Marine Grade Battery Bank As described in sections below |  |  |
| Controls | Control System As described in sections below |  |  |
| Admin | Shipping, Installation and Vessel Commissioning As described in sections below |  |  |
| Training | Operations and Maintenance training As described in sections below |  |  |
| Warranty | As described in sections below |  |  |

### Outboard Motor Specifications for Lot B

See below specifications for the Electric Outboard Motors to be supplied and installed on three (3) new 24ft fiberglass vessels. It is desired for the propulsion systems to have common architecture for ease of maintenance and service.

Table 7 – Electric Outboard Motor Requirements – 24ft vessel

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Continuous Power Rating | Minimum acceptable: 85 kW Maximum: 115 kW |  |  |
| Torque | Minimum acceptable: ≥ 275 Nm Include a torque curve that includes continuous and peak torque capabilities. Continuous means 60 minutes of operation. |  |  |
| Dimension limitations | Dimensions must be comparable to those of common gasoline outboard engine brands such as Yamaha, Suzuki, Honda, Mercury Marine, Tohatsu of comparable power ratings. The electric propulsion unit must be compatible with industry-standard transom mounting configurations and must not exceed the standard physical clearance envelopes of a comparable 100–150 HP gasoline outboard engine." |  |  |
| Motor Efficiency | Motor should be sized appropriately to comfortably achieve the operational range of approximately 30 Nautical miles and based on the proposed battery and general vessel operational parameters. Provide efficiency map and QA tests for the outboard motor so that this can be validated by the technical evaluation committee. |  |  |
| Material | System must be designed for ocean/saltwater use. IP 67 ingress rating or better. |  |  |
| Mounting | Standard outboard mounting required per ABYC TH-12 Standard or ISO 12215-9 series standards. |  |  |
| Propellor | Propeller match system and must perform ≥5000 before replacement required |  |  |
| Safety or Performance Standards | System must meet current ABYC standards including E-30 and A-31 or equivalent standard. All standards must be listed. Only internationally recognised standards will be accepted |  |  |
| Motor Warranty | Warranty period measured from acceptance of vessel (signed “Certificate of Acceptance”). Minimum of a 2-year warranty according to “General Purchase Conditions of GIZ” plus an additional manufacturer guarantee of minimum 1-year must be provided on all core manufacturer-supplied components of the outboard unit. Including but not limited to: - The Electric Motor & Power Electronics: Stator, rotor, internal bearings, and the integrated motor controller/inverter unit housed within the outboard enclosure. - The Drivetrain & Lower Unit: The main drive shaft, internal gearcase components, propeller shaft, and all factory-installed internal water/oil seals. Integrated Auxiliary Systems: Factory-installed internal cooling systems (including integrated pumps and heat exchangers) and the power trim/tilt hydraulic assembly and actuator." |  |  |

### Marine Battery Specifications for Lot B

Table 8 – Marine Battery Specifications – Vessel Propulsion Systems Lot B

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Battery Capacity | ≥ 110 kWh for each of the three (3) stand-alone propulsion systems. Please state size of the battery packs proposed. |  |  |
| Expandable Battery Pack | Battery pack should be capable of expansion by addition of additional packs in parallel to increase capacity |  |  |
| Enclosure Protection Rating | IP67 or better enclosure |  |  |
| Fire Rating | A0 or better fire rating |  |  |
| Marine Standards | ABYC E-30 and A-31 or equivalent standard. Only internationally recognised standards will be accepted. |  |  |
| Battery Thermal and Safety Management System | Battery should have active thermal and safety management system to maintain battery temperature in optimal temperature range appropriate for operation in Tropical Caribbean Climate. An overview of the thermal management system shall be provided, with a reference to the standards applied. The thermal management system must meet the requirements of an Internationally recognized standard issued by IMO Recognized Organization. |  |  |
| Sensors | Thermal monitoring sensors, smoke sensors and fire sensors are required. |  |  |
| Fire Control and Management Procedure | Fire management and emergency procedures must be described. |  |  |
| Battery Warranty | Warranty period measured from acceptance of vessel (signed “Certificate of Acceptance”). Minimum of a 2-year warranty according to “General Purchase Conditions of GIZ” plus an additional manufacturer guarantee of minimum 1-year must be provided on all core manufacturer-supplied components of the outboard unit. Including but not limited to: - The Battery Management System (BMS): All internal and external BMS hardware, balancing circuitry, temperature/voltage sensors, and proprietary control software/firmware. - Cells and Structural Modules: All individual lithium cells, internal busbars, module interconnects, and the sealed, waterproof (IP-rated) battery enclosure. - Safety and Switching Components: Internal high-voltage contactors, fuses, manual disconnect switches, and integrated thermal management components (e.g., internal cooling plates or heat exchangers). |  |  |

### Control System Specifications for Lot B

Table 9 – Control System Specifications Lot B

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Steering wheel, throttle control | Provide an electronic steering wheel and electronic throttle for installation at the helm position. Speciality cables or connectors for connecting the throttle and steering wheel to the propulsion system motor shall be supplied with the system. |  |  |
| Displays and monitoring devices | Speed Guage Battery State of Charge Battery Voltage Monitor |  |  |
| Back-end safety data | Provide data that describes the health of the system for ongoing monitoring. E.g. - Voltage - Faults - SoC - Battery Temperature - Any other critical parameters |  |  |
| Any other specifications | Describe safety standards being met by system with respect to USCG standards. |  |  |

## Propulsion Systems Installation and Commissioning Lot B

The three (3) propulsions systems must be installed on site in Grenada by company representatives. Commissioning of the system must be completed by company representatives. Requirements for training of the local team to complete the installation and commissioning is defined in section 8. The proponent should outline clearly what will be required to complete the installation based on the details provided in section 5.2.1.

### Physical Details for 24-ft vessels to be Outfit with Electric Propulsion in Lot B

Three (3) full power systems (≥100 kW) will be installed on the 24-foot vessels as arranged in Figure 1.

Figure 1 - 24 Foot Vessel General Arrangement

### Positioning and Mounting of Battery and other components Lot B

Batteries should be mounted in a position to maximise operational space for the operations team and generally in the spaces identified in the vessel general arrangements. Any requirements for ventilation and safety of the batteries such as ventilation fans and/or fire suppression systems when installed must be clearly described. Costs related to this must be estimated.

Mounting must be secure, and an anti-theft system/procedure supplied (e.g. Locks, remote alarms). Mounting must allow the system to be operated with ease. Mounting must allow the system to be removed if required.

Please describe any standards the installation will be compliant to.

### Considerations for Electrical and Communications Installation Lot B

If wiring is required, it must be completed with marine grade materials and work oversight and approved by a CVQ (Caribbean Vocational Qualification) certified Level 4 or Level 5 Electrical Installation expert with training in Marine Electrical systems, OR personal with an Advanced Marine Electrical Certification from ABYC and/or certified by the NMEA in Marine Electronics Installation (MEI).

### Provision of Boat Hulls for Installation of Propulsion Systems (Lot B)

The boat hulls, for the propulsion systems procured under Lot C, will be available to the supplier when the commission/installation of the propulsion systems takes place by the supplier in Grenada.

# Lot C: Electric Propulsion Systems for 20-ft Vessels (3 Units)

GIZ seeks to procure all components required for three (3) electric propulsion systems to be installed on newly built vessels used for fishing and transporting people. # systems will be used on three (3) on 20ft vessels. The different requirements for each vessel length are described in the sections below. The vessels will be used primarily for fishing. Fishing vessels require low-speed trolling for 6-8 hours at under 6 knots speed, with short distance, high-speed transit legs at up to 15 Knots speed (typically less than 5 nautical miles from shore). The vessels will be newly constructed, and the following systems will be needed for installation on the vessels:

### General Requirements for Electric Propulsion Systems for Lot C

Table 10 - General Requirements for Propulsion Systems for Lot C

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Propulsion | One (1) Electric Outboard Motor As described in sections below |  |  |
| Energy | Marine Grade Battery Bank As described in sections below |  |  |
| Controls | Control System As described in sections below |  |  |
| Admin | Shipping, Installation and Vessel Commissioning As described in sections below |  |  |
| Training | Operations and Maintenance training As described in sections below |  |  |
| Warranty | As described in sections below |  |  |

### Electric Outboard Requirements for Lot C

Table 11 – Electric Outboard Motor Requirements for Lot C – 20ft vessel

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Continuous Power Rating | Minimum acceptable: 50 kW. Maximum: 60 kW may be software or firmware-governed to 60 kW (80 HP) continuous power rating for the 20ft fleet vessels. |  |  |
| Torque | Minimum acceptable: ≥ 275 Nm Include a torque curve that includes continuous and peak torque capabilities. Continuous means 60 minutes of operation. |  |  |
| Dimension limitations | Dimensions must be comparable to those of common gasoline outboard engine brands such as Yamaha, Suzuki, Honda, Mercury Marine, Tohatsu of comparable power ratings. The electric propulsion unit must be compatible with industry-standard transom mounting configurations and must not exceed the standard physical clearance envelopes of a comparable 100–150 HP gasoline outboard engine." |  |  |
| Motor Efficiency | Motor should be sized appropriately to comfortably achieve operational range of approximately 30 Nm and based on the proposed battery and general vessel operational parameters. Provide efficiency map, QA tests or other supporting data for the outboard motor so that this can be validated by the technical evaluation committee. |  |  |
| Material | System must be designed for ocean/saltwater use. IP 67 ingress rating or better. |  |  |
| Mounting | Standard outboard mounting required per ABYC TH-12 Standard or ISO 12215-9 series standards. |  |  |
| Propellor | Propeller match system and must perform ≥5000 before replacement required |  |  |
| Safety or Performance Standards | System must meet current ABYC standards including E-30 and A-31 or equivalent standard. All standards must be listed. Only internationally recognised standards will be accepted |  |  |
| Motor Warranty | Warranty period measured from acceptance of vessel (signed “Certificate of Acceptance”). Minimum of a 2-year warranty according to “General Purchase Conditions of GIZ” plus an additional manufacturer guarantee of minimum 1-year must be provided on all core manufacturer-supplied components of the outboard unit. Including but not limited to: - The Electric Motor & Power Electronics: Stator, rotor, internal bearings, and the integrated motor controller/inverter unit housed within the outboard enclosure. - The Drivetrain & Lower Unit: The main drive shaft, internal gearcase components, propeller shaft, and all factory-installed internal water/oil seals. - Integrated Auxiliary Systems: Factory-installed internal cooling systems (including integrated pumps and heat exchangers) and the power trim/tilt hydraulic assembly and actuator." |  |  |

### Marine Battery Specifications for Lot C

Table 12 – Marine Battery Specifications for Lot C – Vessel Propulsion Systems

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Battery Capacity | ≥ 110 kWh for each of the six (3) stand-alone propulsion systems. Please state size of the battery packs proposed. |  |  |
| Expandable Battery Pack | Battery pack should be capable of expansion by addition of additional packs in parallel to increase capacity |  |  |
| Enclosure Protection Rating | IP67 or better enclosure |  |  |
| Fire Rating | A0 or better fire rating |  |  |
| Marine Standards | ABYC E-30 and A-31 or equivalent standard. Only internationally recognised standards will be accepted. |  |  |
| Battery Thermal and Safety Management System | An overview of the thermal management system shall be provided, with a reference to the standards applied. The thermal management system must meet the requirements of an Internationally recognized standard issued by IMO Recognized Organization. |  |  |
| Sensors | Thermal monitoring sensors, smoke sensors and fire sensors are required. |  |  |
| Fire Control and Management Procedure | Fire management and emergency procedures must be described. |  |  |
| Battery Warranty | Warranty period measured from acceptance of vessel (signed “Certificate of Acceptance”). Minimum of a 2-year warranty according to “General Purchase Conditions of GIZ” plus an additional manufacturer guarantee of minimum 1-year must be provided on all core manufacturer-supplied components of the outboard unit. Including but not limited to: - The Battery Management System (BMS): All internal and external BMS hardware, balancing circuitry, temperature/voltage sensors, and proprietary control software/firmware. - Cells and Structural Modules: All individual lithium cells, internal busbars, module interconnects, and sealed, waterproof (IP-rated) battery enclosure. - Safety and Switching Components: Internal high-voltage contactors, fuses, manual disconnect switches, and integrated thermal management components (e.g., internal cooling plates or heat exchangers). |  |  |

### Control System Specifications Lot C

Table 12 – Control System Specifications Lot C

| Specification | Requirement | Requirement Met (Yes/No) | Supplier Comments on Requirement |
| --- | --- | --- | --- |
| Steering wheel, throttle control | Provide an electronic steering wheel and electronic throttle for installation at the helm position. Speciality cables or connectors for connecting the throttle and steering wheel to the propulsion system motor shall be supplied with the system. |  |  |
| Displays and monitoring devices | Speed Guage Battery State of Charge Battery Voltage Monitor |  |  |
| Back-end safety data | Provide data that describes the health of the system for ongoing monitoring. E.g. - Voltage - Faults - SoC - Battery Temperature - Any other critical parameters |  |  |
| Any other specifications | Describe safety standards being met by system with respect to USCG standards. |  |  |

## Propulsion Systems Installation and Commissioning Lot C

The three (3) propulsions systems must be installed on site in Grenada by company representatives. Commissioning of the system must be completed by company representatives. Requirements for training of the local team to complete the installation and commissioning is defined in section 8. The proponent should outline clearly what will be required to complete the installation based on the details provided in section 6.2.1.

### Physical Details of 20-ft Vessels to be Outfit with Electric Propulsion Lot C

Three (3) reduced power or governed systems (60 kW) will be installed on vessels as arranged in figure 2.

Figure 2 - 20 Foot Vessel General Arrangement

### Positioning and Mounting of Battery and other components Lot C

Batteries should be mounted in a position to maximise operational space for the operations team and generally in the spaces identified in the vessel general arrangements. Any requirements for ventilation and safety of the batteries such as ventilation and/or fire suppression requirements when installed must be clearly described. Costs related to this must be estimated.

Mounting must be secure, and an anti-theft system/procedure supplied (e.g. Locks, remote alarms). Mounting must allow the system to be operated with ease. Mounting must allow the system to be removed if required.

Please describe any standards the installation will be compliant to.

### Considerations for Electrical and Communications Installation Lot C

If wiring is required, it must be completed with marine grade materials and work oversight and approved by a CVQ (Caribbean Vocational Qualification) certified Level 4 or Level 5 Electrical Installation expert with training in Marine Electrical systems, OR personal with an Advanced Marine Electrical Certification from ABYC and/or certified by the NMEA in Marine Electronics Installation (MEI).

### Provision of Boat Hulls for Installation of Propulsion Systems (Lot C)

The boat hulls, for the propulsion systems procured under Lot C, will be available to the supplier when the commission/installation of the propulsion systems takes place by the supplier in Grenada.

# Training (All Lots)

Vendor must provide operation and maintenance training to the Grenada team for each lot.

## Lot A – Training Requirements

The supplier shall provide a comprehensive practical training programme delivered in Grenada held in a workshop and practical environment and using the procured vessel as the training platform. Training shall cover the major systems of the vessel and include driver and operator training.

Training proposal should include the following assumptions as minimum requirements:

- Conducted in ENGLISH Language

- Duration of at least 1 work week (5 working days and at least 6 hours per day)

- At least 1 technical facilitator with detailed technical knowledge of the products proposed

- To be conducted in Grenada, W.I.

- GIZ or the government of Grenada will organize the venue, logistics, ground transport for instructor(s) and catering in Grenada for training.

- We will assume around 25-30 participants for the training.

  - The participants will include:

    - Coast guard naval officers; technical officers of the blue economy and climate resilience ministry, other experienced sea men & technicians and other persons involved in the project

    - The vessels will be used for low speed near shore patrol by the coast guard and by the blue economy ministry for patrolling marine-protected areas.

- The proponent will need to cover the expenses for the trainer as a lump sum that covers all expenses including but not limited to:

  - Round-trip flights

  - Accommodation

  - Per diems, food and incidentals

  - Any visas or travel expenses required for the trainer

  - Please view “§ 7.1.3 Services at the place of assignment”

### Driver Training (Practical)

The supplier shall provide practical driver training using the Turnkey vessel as the training platform, covering at minimum:

- Start-up/shutdown procedures and safe operating practices for battery electric vessels

- Efficient driving techniques and energy management to maximize range

- Understanding dashboard indications, warnings and fault messages

- Pre-trip inspections and daily checks

- Safe charging procedures (connect/disconnect, basic fault recognition)

- Emergency procedures (safe stop, isolation, incident reporting)

### Operator Training (Operations and Depot)

The supplier shall provide operator training for personnel responsible for day-to-day operations and supervision, covering at minimum:

- Operational planning considerations for battery electric vessels (charging windows, scheduling, range management)

- Basic interpretation of operational data (energy consumption, charging sessions, fault alerts)

- Recommended operational controls to protect battery health (charging practices, temperature management)

- Incident management and escalation procedures (including coordination with maintenance personnel and first responders)

Training deliverables shall include:

- Training agenda and session plans (including time allocations and practical exercises)

- Training materials (handouts) and workshop exercises/checklists, including any required tool list

- Access to maintenance and diagnostic software required for training and maintenance activities (licensing terms to be stated)

- Attendance records and certificates of completion

The supplier has to hand in a training agenda with their offer.

### Services at the place of assignment:

In accordance with the technical specification and tender specifications, services are to be provided at the place of assignment. The respective service offered/applicable for installation, commissioning, approval, instruction, training, etc., must include all the following costs.

- including costs for travel expenses for trainer, other mobilisation and demobilisation costs (e.g. flight expenses, preparation and follow-up costs, documentation, etc.)

- including travel-related expenses, ancillary travel expenses (visas, medical arrangements, etc.), accommodation costs, meal costs, insurance

- including, where applicable, provision and transport of tools, equipment, necessary tools, documents, teaching materials, etc.

- including transport for personnel at the place of assignment

The following will be provided by the project and shall be excluded from offers

- The project will organize the venue, logistics, ground transport for instructor(s)

- transport for instructor(s) at the place of assignment and any transport for training participants on site is provided locally by the project

- food and catering for instructor(s) and training participants during training sessions will be provided by the government of Grenada or GIZ

Additional services and associated reimbursements will be recognised only if these:

- Have become necessary through no fault of the contractor or its suppliers

- The commissioning party is notified immediately

- The commissioning party has recognised such additional services

## Lot B and C – Training requirements

Training for Lot B and C will follow a similar structure to the requirements for Lot A in terms of content. However, it should be assumed that this training is done in conjunction with the installation and vessel commissioning in sections 5.2 and 6.2.

The hulls described in Lot 5.2 and 6.2 will be fabricated and supplied separately from this procurement and will be made available for installation, commissioning and training.

Therefore, this training plan should be incorporated/combined with the plan for installation and vessel commissioning.

# Shipping and Logistics (All Lots)

Equipment should be shipped using the following incoterms:

- CIF Incoterms to Port of St. George’s, Grenada W.I.

Items required for installation will be exempt from import duties and taxes (exemption will be organised by GIZ and the Government of Grenada).

All items should be shipped to Grenada. Onward transport to respective locations will be organized by GIZ or the Government of Grenada.

GIZ and the government of Grenada will work with their broker to clear the items from the port once all the appropriate shipping documents and Bill of Laden are provided to us.

# Project Delivery Times

Please see anticipated project delivery timelines.

### Delivery Time - Lot A

Complete vessel commissioning and handover should be completed within 210 days (7 months of issuing a purchase order.

Vessel commissioning and training should be completed within 30 days of boats arriving in Grenada.

### Delivery Time - Lot B and Lot C

The Equipment should be supplied to Grenada within 90 days (or 12 weeks) of issuing a purchase order.

Installation, vessel commissioning and training should be completed within 180 days of issuing a purchase order and after all hulls have been fabricated. Subject to scheduling from GIZ.

# After-sales support (All Lots)

International suppliers should, where possible, identify local or regional companies or individuals (e.g. within Grenada or the Caribbean) who could provide after-sales support for the equipment supplied. After-sales support shall be provided for a minimum of 2 years after the supply of the equipment according to “GIZ General Purchase Conditions”.

After Sales Support: It is assumed that in this scenario that this would be an approved company who could support in conducting any required preventative maintenance tasks, where required, or provide support with any corrective maintenance tasks that may arise, at minimum. At best, this company should be capable of providing support in any warranty claims processes that may arise.

Any questions or concerns must be promptly answered by the vendor for 6 months after commissioning (after sales).

|  | Requirement | Supplier Comments on Requirement |
| --- | --- | --- |
| Contact Details | Your company name and address. |  |
| 2-year warranty according to GIZ General purchase conditions | Yes / comply |  |
| Workshop Details | Where possible, please provide name and address of a local/regional partner responsible for providing after-sales for the proposed equipment. |  |

# Acceptance

Acceptance of supplier’s work and materials shall take place after completion. Supplier must notify GIZ of the date of completion no later than three weeks in advance.

Please view below the different acceptance procedures for the different lots:

## Acceptance for lot A takes place in two stages:

- The first acceptance test takes place upon receipt of the vessels in Grenada. The project will check if the goods have arrived in good condition and that everything was installed and functioning correctly.

- The second acceptance test will take place once the vessels have been commissioned/installed and the training sessions have been successfully completed.

- The bank guarantee will only be released once both acceptance procedures have been completed and the document “Certificate of Acceptance” is signed. Furthermore, a final invoice must be handed in.

## Acceptance for lots B & C is done according to the following procedure:

- The first acceptance test takes place upon receipt of the propulsion vessels in Grenada. The project will check, if the goods have arrived in good condition and that everything was installed and functioning correctly.

- The second acceptance test will take place once the goods have been commissioned/installed and the training sessions have been successfully completed.

- After the first acceptance, the bank guarantee can be partially released of the value of the propulsion systems itself (not including costs for the other components, like commissioning, installation and training) if, for project-related reasons, the installation and training cannot be completed within 8 weeks after arrival of the propulsion systems in Grenada.

- The bank guarantee will fully be released once both acceptance procedures have been completed and the document “Certificate of Acceptance” is signed. Furthermore, a final invoice must be handed in.

Acceptance tests of the entire work and materials supplied shall be carried out at the place of assignment (Grenada) or by a representative appointed for this purpose in writing by GIZ. The work shall be considered accepted solely upon formal written confirmation thereof by GIZ or its representative appointed for acceptance. The acceptance certificate attached to the tender documents shall be used to confirm acceptance. The entire work and materials shall be put into operation for acceptance purposes.

Please view more details in the document “Special Contractual Terms and Conditions”.